A-JACKS® CONCRETE ARMOR UNIT SPECIFICATION

48” A-Jacks Units

PART 1:	GENERAL

A.	Scope of Work

The Contractor shall furnish all labor, materials, equipment, and incidentals required and perform all operations in connection with the installation of A-JACKS® concrete armor units in accordance with the lines, grades, design and dimensions shown on the Contract Drawings and as specified herein.

B.	Submittal

The Contractor shall submit to the Engineer all manufacturers’ hydraulic testing and calculations in support of the proposed hard armor unit and geotextile.

The Contractor shall furnish the manufacturer's certificates of compliance for the concrete hard armor units. The Contractor shall also furnish the manufacturer's specifications, literature, shop drawings for the layout of the A-JACKS®, and any recommendations, if applicable, that are specifically related to the project.

Alternative materials may be considered.  Such materials must be pre-approved in writing by the Engineer prior to bid date.  Alternative material packages must be submitted to the Engineer a minimum of fifteen (15) days prior to bid date.  Submittal packages must include, as a minimum, the following:

1.	Full-scale laboratory testing and associated engineered calculations quantifying the hydraulic capacity of the proposed hard armor system in similar conditions to the specific project.

PART 2:	PRODUCTS

A.	General

The geometry of an A-JACKS® concrete armor unit consists of six arms extending from a central hub.  A complete unit is made up of two identical halves, with each half consisting of a central core with three legs radiating outward at equal spacing.  On each half, two fillets are located between adjacent arms.  These fillets provide additional structural strength and aid in the proper placement of the armor units.

When the symmetrical halves are interlocked, the resultant unit will have geometry, which exhibits six equally spaced arms, with each arm spaced at 90 degrees from the four adjacent arms.  When placed in the most stable configuration, each unit will rest on three of the six arms.

B.	Concrete Armor Units

1.	Scope

1.1	This specification covers concrete armor units for erosion control used for coastal applications (shoreline, breakwaters, jetties, and other harbor structures), along with toe-protection and slope protection for streambank applications.

- Materials

The 4 ft (AJ-48) A-Jacks® units will be produced on a pre-determined concrete block machine and will conform to ASTM D 6684-04, Standard Specification for Materials and Manufacture of Articulating Concrete Block (ACB) Revetment Systems.

2.1	Cementitious Materials - Materials shall conform to the following applicable ASTM specifications:

2.1.1	Portland Cements - Specification C 150, for Portland Cement.

2.1.2	Blended Cements - Specification C 595, for Blended Hydraulic Cements.

2.1.3	Hydrated Lime Types - Specification C 207, for Hydrated Lime Types.

2.1.4	Pozzolans - Specification C 618, for Fly Ash and Raw or Calcined Natural Pozzolans for use in Portland Cement Concrete.

2.2	Aggregates shall conform to the following ASTM specifications, except that grading requirements shall not necessarily apply:

2.2.1	Normal Weight - Specification C 33, for Concrete Aggregates.

2.3 	Grout– Utilize for assembling 48” A-Jacks Units

- Casting

The concrete units will be produced by a dry cast method.  The dry cast units obtain strength in a shorter duration as well as an increase in the durability and overall quality of product. Material and manufacture standards will be compliant with ASTM D6684-04.

- Physical Requirements

  - At the time of delivery to the work site, the units shall conform to the                        physical requirements prescribed in Table 1 below.

| TABLE 1. A-JACKS® PHYSICAL REQUIREMENTS |  |  |  |
| --- | --- | --- | --- |
| Compressive Strength Net Area Min. psi |  | Water Absorption Max., lb/ft3 |  |
| Avg. of 3 units | Individual Unit (min. required) | Avg. of 3 units | Individual Unit |
| 4000 | 3,500 | 9.1 | 11.7 |

4.2	Units shall be sampled and tested in accordance with ASTM C 140, Standard Test Methods of Sampling and Testing Concrete Masonry Units.

- Visual Inspection

5.1	All units shall be sound and free of defects that would interfere with either the proper placement of the unit or impair the performance of the system.  Surface cracks incidental to the usual methods of manufacture, or surface chipping resulting from customary methods of handling in shipment and delivery, shall not be deemed grounds for rejection.

  - Cracks exceeding 0.25 inches (.635 cm) in width and/or 1.0 inch (2.54 cm) in depth shall be deemed grounds for rejection.

5.3	Chipping resulting in a weight loss exceeding 10% of the average weight of a concrete unit shall be deemed grounds for rejection.

5.4	A-Jacks® rejected prior to delivery from the point of manufacture shall be replaced at the manufacturer's expense.  A-Jacks® rejected at the job site shall be repaired with structural grout or replaced at the expense of the contractor.

- Sampling and Testing

6.1	The purchaser or their authorized representative shall be accorded proper access to facilities to inspect and sample the units at the place of manufacture from lots ready for delivery.

6.2	Field installation procedures shall comply with the procedures utilized during the hydraulic testing procedures of the recommended system. All system restraints and ancillary components (such as synthetic drainage mediums) shall be employed as they were during testing.  For example, if the hydraulic testing installations utilize a drainage layer then the field installation must utilize a drainage layer; an installation without the drainage layer would not be permitted.

6.3	Additional testing, other than that provided by the manufacturer, shall be borne by the purchaser.

- Manufacturer

The A-JACKS® concrete system shall have the following nominal characteristics:

TABLE 2.  STANDARD SIZES OF A-JACKS®

| A-JACK Model |  | Total Length (in) | Arm Length (in) | Fillet Length (in) | Arm Width (in) | Volume (ft3) | Weight (lbs) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AJ-48 | 48 |  | 16.64 | 3.68 | 7.36 | 4.49 | 584-628 |

The Armor unit shall be A-JACKS® as manufactured by:

ARMORTEC, A Contech Company

9025 Centre Pointe Dr., Suite 400

West Chester, OH 45269

Phone:	1-800-645-7000

Fax:	1-513-645-7993

C.  	Filter Fabric

The geotextile will meet the minimum physical requirements listed in Table No. 3 of these Specifications.  Consultation with the manufacturer is recommended.

The geotextile must be permitted to function properly by allowing relief of hydrostatic pressure; therefore fine soil particles will not be allowed to clog the filter fabric.

During all periods of shipment and storage, the filter fabric will be protected from direct sunlight, ultraviolet rays and temperatures greater than 140 degrees Fahrenheit.  To the extent possible, the fabric will be maintained wrapped in its protective covering.  The geotextile will not be exposed to sunlight, ultraviolet rays until the installation process begins.

## TABLE 3: PHYSICAL REQUIREMENTS

| Physical Property | Test Procedure | Minimum Value |
| --- | --- | --- |
| Grab Tensile Strength | ASTM D4632 | 280 Lbs. |
|  |  | (in any principal direction) |
| Breaking Elongation | ASTM D4632 | 50% max. |
|  |  | (in any principal direction) |
| Trapezoidal Tear | ASTM D4533 | 112 Lbs. |
| CBR Puncture Strength | ASTM D6241 | 800 Lbs. |
| A.O.S. | ASTM D4751 | Soil Specific |

Soil characteristics such as grain size distribution and plasticity will be determined for every 200,000 square feet of geotextile installed or for each source of borrow material used during construction.  Significant differences in soil characteristics will require further inspection at the discretion of the Engineer.  The locations for which the material to be tested is extracted will be approved by the Engineer.  The Contractor will provide the site-specific soil and modified proctor curves for the site-soil, at no expense to the manufacturer.  Also, the contractor will be responsible for the performance of the test by a certified independent laboratory experienced in performing such test.  The test will be performed under the actual field soil conditions or as otherwise required by the Engineer.

The need and frequency of soil testing is explicitly at the discretion of the Engineer Record.

At the time of installation, the filter fabric will be rejected if it has been removed from its protective cover for over 72 hours or has defects, tears, punctures, flow deterioration, or damage incurred during manufacture, transportation or storage.  With the acceptance of the Engineer, placing a filter fabric patch over the damaged area prior to placing the mats will repair a torn or punctured section of fabric.  The patch will be large enough to overlap a minimum of three (3) feet in all directions.

D.	Cabling

Termination or transition of the A-Jacks armoring will require additional field cabling. Cable type will be minimum 1/4” galvanized or polyester, as approved by the Engineer.

Field cabling is done to mitigate unexpected local toe scour or to add redundancy at the termination and transition points. Cable will be placed around the mid-section of the selected units (recommended 3-unit minimum), hand tightened and connected using standard cable hardware supplied by the manufacturer.

Cable orientation of the recommended 3-unit minimum (i.e. perpendicular or parallel to a slope) will be identified and agreed upon in the field by the Contractor and Engineer’s representative. Ensure practical placement of additional cabling to minimize unreasonable effort by Contractor. Prior to field cabling work, means and methods will be discussed to establish a typical configuration for the previously identified areas.

Ultimately, the number of units to be cabled as well as orientation is at the discretion of

## Armor Unit Performance Specifications –Channel Bed and Bank Protection

D.1	The armor unit should provide sufficient hydraulic stability under the design conditions specified above.

D.2	The Factor-of-Safety (FOS) Method should be used in evaluating the hydraulic stability of the chosen armor units.

D.3	The FOS for shear stress, SF, should be greater than 2, where SF is defined in the following:

SF = 	p/o

where p is the permissible stress for the armor unit accounting for the appropriate side slope correction factor, and o is the maximum bed shear stress corresponding to the design condition.

D.4	The FOS for velocity, SFv, should be greater that 2, where SFv is defined in the following:

SFv =  Vp/V

where Vp is the permissible velocity for the armor unit accounting for the appropriate side slope correction factor, and V is the design velocity.

PART 3:	FOUNDATION, GEOTEXTILE AND A-JACKS PREPARATION & PLACEMENT

A.	Foundation Preparation

General. All subgrade preparation should be performed in accordance with ASTMD6884-03, Standard Practice for Installation of Articulating Concrete Block (ACB) Revetment Systems.

Areas on which filter fabric and the Hard Armor Units are to be placed will be constructed to the lines and grades shown on the Contract Drawings and to the tolerances specified in the Contract Documents, and approved by the Engineer.

Grading.  The slope will be graded to a smooth plane surface to ensure that intimate contact is achieved between the slope face and the geotextile (filter fabric). All slope deformities, roots, grade stakes, and stones which project normal to the local slope face must be re-graded or removed.   Where such areas are evident, they will be brought to grade by placing compacted homogeneous material.  The slope and slope face will be uniformly compacted, and the depth of layers, homogeneity of soil and amount of compaction will be as required by the Engineer.

Inspection.  Immediately prior to placing the filter fabric and remaining subgrade components, the prepared subgrade will be inspected by the Engineer as well as the owner's representative.  No fabric, subgrade section or Hard Armor Unit will be placed thereon until that area has been approved by each of these parties. “Rules of thumb” to be established in initial inspection of each product type with regard to foundation preparation to facilitate efficiency in future project preparation areas.

B.	Placement of Geotextile Filter Fabric

General.

All placement and preparation should be performed in accordance with ASTM D6884-03, Standard Practice for Installation of Articulating Concrete Block (ACB) Revetment Systems. Filter Fabric, or filtration geotextile, as specified elsewhere, will be placed within the limits shown on the Contract Drawings.

Placement.  The filtration geotextile will be placed directly on the prepared area, in intimate contact with the subgrade, and free of folds or wrinkles.  The geotextile will not be walked on or disturbed when the result is a loss of intimate contact between the geotextile and the subgrade.  The geotextile filter fabric will be placed so that the upstream strip of fabric overlaps the downstream strip. The longitudinal and transverse joints will be overlapped at least two (2) feet.  The geotextile will extend at least one (1) foot beyond the top and bottom revetment termination points, or as required by the Engineer.

C.	Placement of A-Jacks System

The proposed A-Jacks units, as specified in Part 2A of these specifications, will be constructed within the specified lines and grades shown on the Contract Drawings.

Placement (No. 57 Stone) – 48” A-Jacks

If applicable, to mitigate subgrade inconsistencies, a minimum thickness of 4” of No. 57 stone is required prior to installing the 48” units. The stone and desired thickness will be placed directly on the specified geotextile as directed and accepted by the Engineer of Record or representative.

Bedding – 48” A-Jacks

For both A-Jack sizes, a bedding layer of clean stone is to be placed in conjunction with the subgrade section for backfilling purposes to achieve necessary bearing and/or setback tolerances. Recommended bedding layer sizes for each size unit are listed below:

| A-Jacks Size (in.) | D50 Size Bedding Layer (in.) |
| --- | --- |
| 48" | 4-6" |

Alternate D50 sizes for bedding layers will be defined by local aggregate availability, approved by the Design Engineer.

Placement – 48” A-Jacks

A-Jacks can be installed individually or in bundles at the discretion of the Contractor.

Placed individually, A-Jacks will be placed with uniform spacing. For a specified area receiving the 48” units, standard spacing between unit center in both the x and y dimension is 24”, which represents a 0.5 packing ratio. Tighter placement tolerances are possible, such as a 0.4 packing ratio, but may become difficult to achieve while placing successive rows in a matrix. Standard installation includes placing the units in a consistent repeatable fashion to aid in efficiency. Recommended in-place orientation of the unit(s) emphasizes pointing exposed projecting unit arm (vertical or horizontal) downstream whenever practical during construction.

Placed in bundles, A-Jacks are placed into a pre-determined matrix, spacing them as closely as possible, approximating the maximum packing ratio of 0.4 (19.2” center spacing in x and y directions). Cable will be placed around the pre-determined matrix (around mid-section) with cable; hand tightened and connected using standard cable hardware supplied by the manufacturer. Bundles are to be placed as closely as possible, with recommended tolerance requirement of 8” between bundles. If the bundling method is employed by the Contractor, tolerances between bundles will be discussed and agreed upon prior to commencing work. Spacing will be measured from center of bundle to middle of installed bundle, in the horizontal plane, before additional (stacked) units are placed.

Bedding stone material can be utilized for bridging interior voids and establishing consistent bedding layer for additional lifts of unit(s), whether hand placed or bundles. Vertical fluctuations occurring from unit to unit or bundle to bundle will be compensated by utilizing bedding stone.

Consultation.  The manufacturer of the hard armor units will provide design and construction advice during the design and initial installation phases of the project as necessary, by the discretion of the Engineer.